Division 1 – General

1.01 Section Includes:

- Steel Unassembled (Knock Down) Lockers

1.02 References

- Americans with Disabilities Act – Accessibility Guidelines

1.03 Shop Drawings, Material Data, Samples, and Product Data (Submittals)

- Submit under provisions of Section 01 33 00.

- Manufacturer data sheets used on products:

- Construction and Reference Instructions

- Requirements and Recommendations for Storage and Handling

- Install Procedures

- Submittals – Include the Following:

- Dimensional Drawings: Plans, sections, and elevations to illustrate locker placement and interfaces with neighboring materials.

- Specifics of Assembly Requirements

- Collection Examples: For each individual finish product, two comprehensive sets of color chips which represent the manufacturer’s full range of existing colors and finishes.

1.04 Storage, Distribution, and Handling

- Store products in unopened manufacturer’s packaging until prepared to install.

- Shield locker and adjacent surfaces from damage.

Division 2 – Products

2.01 Manufacturers

- Acceptable Manufacturer: Jorgenson Lockers, which is located at 1239 S. 700 W., Salt Lake City, UT 84104; Tel: 800-952-0151; Fax: 801-493-0158; Email: quotes@jorgensonlockers.com; Web: www.jorgensonlockers.com

- Substitutions: Not Permitted.

- Appeals for substitutions will be considered in accordance with requirements of Section 01 25 00.

2.02 Materials

- Steel: Prime grade mild cold-rolled sheet steel, free from surface flaws.  Must be compliant with ASTM A1008 and capable of taking a powder-coated enamel finish.

- Steel: Sheet steel components are fabricated using zinc-coated steel, required to be free from external imperfection, capable of taking a powder-coated enamel finish. Must be compliant with ASTM A879.

- Hooks: Forged, Zinc-plated steel with ball ends.

- Nuts and Bolts: Truss fin head bolts and hex nuts, zinc-plated.

2.03 Standard Gladiator Lockers

- Standard Duty Lockers:

- Acceptable Product: Jorgenson Gladiator, knock-down lockers.

- 16-gauge sheet steel tops, bottoms, sides, and shelves.

- 18-gauge steel backs.

- 14-gauge front frame.

- Locker Body: Steel exclusively formed for additional strength and structure. This ensures tight joints at fastening points.

- Tops and bottoms with three sides bent at 90 degrees, the front formed with an offset to tuck under the horizontal member of the frame.

- Four-sided shelves bent to 90 degrees, front edge includes a second bend.

- Hole spacing in the construction of locker body: Not to surpass 9 inches.

- Front Frame members must be formed to a channel shape from no less than 16-gauge steel.

- Interlocking mortise and tenon parts; electrically welded in a rigid frame that is capable of repelling strains.

- Securely welded cross frame members of channel shapes to vertical frame members to ensure stability.

- Optional factory assembly of locker with steel rivets.

2.05 Interior Equipment

- Hooks are made of zinc-plated forged steel with ball ends.

- 1 coat rod and 2 wall hooks come standard.

- Four-sided shelves bent to 90 degrees, front edge includes a second bend.

- Optional add-on interior equipment:

  - Lockable Security Box

- Located between the top shelf and top of locker

- Door features a recessed cup handle and 2 five-knuckle hinges. Hinges are 2 inches tall and .074 inches thick

  - Lockable Footlocker

  - 14-gauge steel

  - Ventilated front.

  - Hinged seat/hatch with continuous hinge

  - Padlock loop

  - Ball Storage Retainer

- Top bar: 14-gauge

- Elastic cord

2.06 Accessories

- Number Plates: Provide each locker with a polished aluminum number plate, 2 ¼” inch wide by 1” inch high. Includes black numerals not less than 3/8” inch high; attached to using two rivets.

- Locks: Built-in flat key locks with master-key to same series.

- Locks: Built-in grooved key locks (pin tumbler) with master-key to same series.

- Locks: Built-in three-number dial combination locks. Each lock has five distinct, changeable combinations. Provide master key, combination change key, and combination control charts.

- Padlocks: Master-keyed three-number dialing combination type padlocks; provide master key. Mechanism must be resistant to “shimming”

- Coin-Operated Locks:

1. Deposit and Coin Return Type

- Token

- One Quarter

- Two Quarters

2. Coin Collect/Pay Type with Cash Box

- Token

- One Quarter

- Two Quarters

- Front Fillers: 20-gauge steel molded to an angle shape. Includes 20-gauge slip joint angles formed at an angle shape with double bend on one leg forming a pocket to provide adjustable mating with angle filler.

- Attached with concealed fasteners.

- Finish must match lockers.

- Zee Bases for Knock-Down Lockers: 14-gauge steel. Flanged outward at top to support lockers, flanged inward at bottom for securing to floor.  Available in 4 or 6 inch heights.

- Recess Trim: 18-gauge steel, 3 inch face dimension.

- Vertical and/or horizontal as required.

- Standard lengths as long as practical.

- Attaches to lockers with hidden clips.

- Provide needed finish caps and splices.

- Finish must match lockers.

2.07 Fabrication

- Construct lockers to be free from distortion. The units must be square, rigid, devoid of warp, and with flat metal faces.

- Confirm measurements and arrangement before construction.

- Finish: Enamel powder coat paint finish electrostatically applied and properly cured to manufacturer’s specifications. No finishes containing volatile organic compounds and subject to out-gassing will be accepted. The color of the interior and exterior of the locker must match.

- Powder Coat-Dry Thickness: 1 to 1.2 mils.

- Powder Coat Plus-Dry Thickness: 2 to 2.2 mils.

  - Color: As selected from manufacturer’s standard colors.

- Special Finishes

- Anti-Microbial

- Anti-Graffiti

- TGIC

- Resistant to Weather

- Custom Colors.

Division 3 – Execution

3.01 Examination

- Do not begin set-up until substrates and bases have been correctly prepared.

- If substrate and bases are the responsibility of a third party, notify architect of unsatisfactory preparation before proceeding.

3.02 Installation

- Install metal lockers and accessories at locations shown, per agreement with manufacturer’s instructions.

- Lockers must be installed plumb, level, and square.

- Anchor lockers to the floor and wall at 48 inches (1.219 m) or less, follow the manufacturer recommendation.

- Bolt adjoining units together to provide rigid installation.

- Install metal fillers using concealed fasteners. Provide even and flat hairline joints against neighboring surfaces.

3.03 Cleaning and Adjusting

- Adjust doors and latches to function without binding. Confirm that latches are functioning acceptably.

- Adjust built-in locks to avoid binding of dial or key. This will keep operation smooth prior to substantial completion.

- Fix flaws with factory-supplied paint and mend/replace damaged products before substantial completion.

3.04 Protection

- Shield installed products until completion of task.